VITAL AIR — Delhi NCR Air-Quality Forecasting & Decision Support System

Complete Technical Architecture, Folder Structure, Frontend & Backend Stage 1–11 Comprehensive Reference

# 1. Executive Summary & System Overview

VITAL AIR is a state-of-the-art atmospheric science, machine learning, and decision-support system designed specifically for the Delhi National Capital Region (NCR). The project models complex boundary-layer meteorology, regional transport, multi-sector anthropogenic emissions, satellite-detected stubble burning, WRF-Chem aerosol-radiation feedback, 24/48/72-hour forecasting, and automated emergency interventions (GRAP mandates).

NOTE: Core Physical Paradigm: The Dual-Loop Mechanism 1. Forward Loop (Meteorology -> Chemistry): Boundary layer height (PBLH), solar radiation, temperature, and wind fields drive spatial transport, turbulent mixing, and photolysis reactions (O3-NOx-CO-aerosol production). 2. Reverse Loop (Aerosol -> Meteorology Feedback): Aerosols dim surface shortwave radiation (SWDOWN), lowering surface temperature (T2), suppressing PBL development (PBLH), reducing vertical mixing, and trapping PM2.5 near the ground.

# 2. Master Project Directory Architecture

The codebase is strictly structured into modular components to ensure clean separation between UI visualization, physical modeling stages, shared data repositories, and operational API layers:

| Folder / Directory | Component Role | Primary Responsibilities & Key Files |
| --- | --- | --- |
| Frontend/ | User Interface | React 18 Vite app, Tailwind/Vanilla CSS styling, interactive GIS maps, live telemetry panels, GRAP mandate banners, and analytical modals. |
| Backend/stage1/ | Stage 1 Prototype | Python dual-loop prototype (dual_loop_air_quality.py) with forward transport-chemistry and reverse aerosol optical depth feedback. |
| Backend/stage2/ | Stage 2 Met Data | Real meteorological data reader (netcdf_reader.py, main_real_data.py) ingesting ERA5/WRF T2, U10, V10, PBLH, SWDOWN. |
| Backend/stage3/ | Stage 3 Validation | Ground air quality observations reader (air_quality_reader.py) and validation metrics engine (MAE, RMSE, Bias, R, R2). |
| Backend/stage4/ | Stage 4 Spatial Transport | 2D spatial finite-difference advection-diffusion solver over a 20x20 Delhi NCR grid (grid_model.py, transport_model.py, main_stage4.py). |
| Backend/stage5/ | Stage 5 Emissions | Multi-sector anthropogenic emission inventory pipeline (traffic, industry, domestic) with SI unit conversion to kg/s. |
| Backend/stage6/ | Stage 6 Fire Emissions | Satellite active fire detection processing (VIIRS/MODIS), FRP integration, agricultural stubble filter, and emission factor calculation. |
| Backend/stage7/ | Stage 7 WRF Met Model | WRF/WPS numerical weather prediction configuration templates (geogrid, ungrib, metgrid, real.exe, wrf.exe) for Delhi NCR. |
| Backend/stage8/ | Stage 8 WRF-Chem | WRF-Chem atmospheric chemistry workflow (RADM2/CBMZ gas-phase, MADE/MOSAIC aerosols), speciation mapping, and NetCDF output reader. |
| Backend/stage9/ | Stage 9 Feedback | Two-way aerosol-meteorology coupling suite comparing CONTROL (Feedback OFF) vs FEEDBACK (Feedback ON) numerical experiments. |
| Backend/stage10/ | Stage 10 Forecasting | Automated 24/48/72-hour regional air quality forecasting pipeline, fire decay projection, SQLite storage (forecast.db), and horizon validation. |
| stage11_dashboard/ | Stage 11 Production | Operational web application layer featuring FastAPI backend (api/main.py), Streamlit dashboard (dashboard/app.py), and Docker containerization. |
| Backend/data/ | Shared Data Repository | Standardized directories storing meteorology, ground observations, satellite fires, emission inventories, and WRF-Chem NetCDF datasets. |
| Backend/outputs/ | Shared Outputs Store | Output CSV statistics tables, horizon verification metrics, SQLite databases, and PNG spatial visualization maps. |

# 3. Frontend System Architecture (Frontend/)

The frontend application provides a high-density, real-time command dashboard for environmental decision-makers, policy authorities, and atmospheric researchers. Built using React 18, Vite, and custom SVG/GIS map rendering, it presents real-time telemetry, spatial plumes, satellite fire hotspots, and forecast outlooks.

## 3.1 Core Components & Layout Modules

| Component File | Function & UI Telemetry Provided |
| --- | --- |
| VayuCommandDashboard.jsx | Primary command dashboard layout assembling headers, telemetry panels, GIS maps, and GRAP mandate banners. |
| TopHeaderBar.jsx & CommandHeader.jsx | Displays real-time system clock, operational status indicators, active AQI ticker, and GRAP emergency status. |
| RegionalAqiHeroPanel.jsx | Displays current regional AQI, primary pollutant concentration (PM2.5/PM10), and severe exposure warnings. |
| LiveMapPanel.jsx & CenterMapPanel.jsx | Interactive GIS map featuring satellite stubble fire hotspots, wind vector overlays, monitoring station markers, and plume heatmaps. |
| ChemicalTelemetryPanel.jsx | Real-time concentration gauges and time-series trends for PM2.5, PM10, NO2, O3, CO, and SO2. |
| BoundaryLayerMetPanel.jsx | Displays planetary boundary layer height (PBLH), surface 2m temperature, relative humidity, and solar radiation dimming. |
| SpatialSectorsPanel.jsx | Sectoral source attribution breakdown showing relative contributions from traffic, industry, domestic, and stubble fires. |
| TrajectoryOutlookPanel.jsx | 24/48/72-hour pollutant trajectory outlook graphs with lead-time uncertainty bands. |
| GrapMandateBanner.jsx & ImmediateInterventionsPanel.jsx | Automated Graded Response Action Plan (GRAP) emergency mandates (truck entry bans, industrial shutdowns, sprinkling). |
| DeepInsightsModal.jsx | In-depth analytical modal presenting cross-correlation charts, diurnal cycles, and model validation statistics. |
| MapExpandModal.jsx | Full-screen interactive GIS map view for detailed spatial inspection of fire plumes and station networks. |
| services/api.js & store/sensorStore.js | Asynchronous HTTP fetch service connecting UI components to stage11_dashboard FastAPI endpoints and managing global state. |

# 4. Backend Scientific Architecture (Stages 1–10)

The scientific backend is organized into 10 modular stage directories inside Backend/. Each stage represents a complete, self-contained milestone that can be executed independently.

## Stage 1 — Dual-Loop Prototype

Location: Backend/stage1/
Key Files: dual_loop_air_quality.py

Introduces the two-way coupled meteorology-chemistry mechanism in Python. Implements the forward transport/chemistry loop and the reverse aerosol radiative dimming loop. Models Arrhenius photolysis rates, diurnal solar radiation, boundary layer growth/decay, and pollutant accumulation limits.

## Stage 2 — Real Meteorological NetCDF Integration

Location: Backend/stage2/
Key Files: netcdf_reader.py, generate_sample_weather_nc.py, main_real_data.py

Connects real-world ERA5 / WRF NetCDF weather data (T2, U10, V10, PBLH, SWDOWN). Extracts spatial averages, fits diurnal temperature/wind/PBL profiles, and evaluates transport under actual atmospheric conditions.

## Stage 3 — Air Quality Observations & Validation

Location: Backend/stage3/
Key Files: air_quality_reader.py, validation_model.py, generate_sample_aq_csv.py, main_stage3.py

Ingests CPCB ground monitoring station data (PM2.5, PM10, NO2, O3, CO). Standardizes timestamp formats, performs timestamp alignment using pd.merge_asof, and calculates MAE, RMSE, Bias, Correlation (R), and R2.

## Stage 4 — 2D Spatial Atmospheric Transport

Location: Backend/stage4/
Key Files: grid_model.py, spatial_meteorology.py, transport_model.py, spatial_chemistry.py, spatial_feedback.py, main_stage4.py

Upgrades the regional box model into a 2D Eulerian spatial model over a 20x20 grid (5km resolution, 100x100 km Delhi NCR domain). Features a 2D finite-difference advection-diffusion solver, spatial wind vector transport, 2D O3-NOx photochemistry, and 2D spatial aerosol feedback.

## Stage 5 — Multi-Sector Spatial Emission Inventory

Location: Backend/stage5/
Key Files: emission_units.py, emission_gridding.py, traffic_emissions.py, industrial_emissions.py, biomass_emissions.py, emission_inventory.py, main_stage5.py

Unified anthropogenic emission inventory pipeline. Converts emission rates from various units (kg/hour, g/s, kg/day) to SI standard kg/s, allocates point/line/area sources across the 2D grid, and formats outputs for WRF-Chem preprocessors.

## Stage 6 — Satellite Active Stubble Burning Emissions

Location: Backend/stage6/
Key Files: satellite_fire_reader.py, fire_filter.py, frp_model.py, biomass_burned_model.py, emission_factors.py, biomass_emission_model.py, fire_gridding.py, main_stage6.py

Ingests VIIRS / MODIS active fire detections. Applies agricultural stubble-burning land masks, integrates Fire Radiative Power (FRP) to estimate biomass burned, computes pollutant emissions (PM2.5, PM10, CO, NOx, VOC), and injects fire plumes into the spatial grid.

## Stage 7 — WRF Numerical Weather Prediction Model

Location: Backend/stage7/
Key Files: config/ (namelist.wps, namelist.input), scripts/, python/

WRF/WPS numerical meteorology configuration for Delhi NCR (61x61 horizontal domain, 45 vertical levels). Sets up geogrid, ungrib (GFS/ERA5 driver), metgrid, real.exe, and wrf.exe with WSM6 microphysics, RRTMG radiation, and MYNN PBL schemes.

## Stage 8 — WRF-Chem Atmospheric Chemistry & Aerosols

Location: Backend/stage8/
Key Files: config/, scripts/, emissions/ (convert_stage5.py, convert_stage6.py, prepare_emissions.py), python/, main_stage8.py

WRF-Chem atmospheric chemistry integration. Implements RADM2/CBMZ gas-phase chemistry, MADE/MOSAIC aerosols, Fast-J photolysis, emission speciation mapping, WRFChemReader NetCDF parsing, and automated validation against ground observations.

## Stage 9 — Advanced Two-Way Aerosol-Meteorology Feedback

Location: Backend/stage9/
Key Files: config/ (namelist_feedback_off, namelist_feedback_on), scripts/, python/ (aerosol_optics.py, statistics.py, maps.py), main_stage9.py

Quantifies two-way aerosol-meteorology coupling by comparing CONTROL (Feedback OFF) vs FEEDBACK (Feedback ON) WRF-Chem numerical experiments. Demonstrates shortwave radiation dimming (-4.43 W/m2 mean, -67.4 W/m2 peak), surface cooling (-0.076 K mean, -1.16 K peak), PBL suppression (-9.5 m mean, -144.4 m peak), and pollutant trapping (+1.50 ug/m3 mean, +22.87 ug/m3 peak).

## Stage 10 — 24/48/72-Hour Air Quality Forecasting Pipeline

Location: Backend/stage10/
Key Files: config/ (forecast_config.py), ingestion/, emissions/ (forecast_emissions.py), forecast/, wrfchem/, validation/, database/ (forecast_storage.py), main_stage10.py

Operational forecasting engine. Implements persistent fire decay forecasting, 72-hour regional pollutant prediction generation, SQLite relational database storage (outputs/forecast.db), and horizon-based validation across 24h, 48h, and 72h lead times.

# 5. Stage 11 Production Dashboard & Operational API (stage11_dashboard/)

Stage 11 operationalizes the validated WRF-Chem 24/48/72-hour forecasting pipeline into a production-ready software suite featuring a RESTful API backend, interactive web application dashboard, automated GRAP alert engine, and Docker containerization.

| Module / File | Operational Role & Features |
| --- | --- |
| api/main.py | FastAPI backend exposing REST endpoints: /api/forecast, /api/observations, /api/satellite-fires, /api/alerts, and /api/health with CORS & Swagger docs. |
| dashboard/app.py | Interactive Streamlit dashboard presenting live telemetry, forecast charts, satellite fire maps, and emergency alert banners. |
| alerts/alert_engine.py | Automated Graded Response Action Plan (GRAP) alert engine evaluating PM2.5/PM10 thresholds and generating Stage I–IV mandates. |
| database/sqlite_db.py & postgres_schema.sql | Dual-database storage layer supporting SQLite for local runs and PostgreSQL/PostGIS schema for cloud deployments. |
| processing/ (fire_impact.py, metrics.py, quality_control.py, uncertainty.py) | Post-processing pipeline carrying out fire attribution calculation, data quality checks (NaN/Inf filter), metric computation, and ensemble uncertainty bounds. |
| Dockerfile & docker-compose.yml | Production containerization setup exposing FastAPI (port 8000) and Streamlit (port 8501) with python:3.10-slim base. |
| run_pipeline.py & run_alerts.py | Master orchestration scripts running continuous ingestion, WRF-Chem forecast processing, DB updates, and alert dispatch. |

# 6. Summary of Generated Datasets & Artifacts

The following table summarizes all primary CSV data records, NetCDF atmospheric files, SQLite databases, and PNG visualization figures generated across the project stages:

| Artifact Name | Stage Source | Contents & Key Metrics |
| --- | --- | --- |
| dual_loop_results.csv | Stage 1 | 24-hour simulation timeseries of PM2.5, PM10, O3, NO2, and PBL height. |
| processed_weather.csv | Stage 2 | Hourly extracted ERA5/WRF meteorology (T2, U10, V10, PBLH, SWDOWN). |
| model_observation_comparison.csv | Stage 3 | Model predictions vs CPCB ground monitoring station observations. |
| spatial_model_results.csv | Stage 4 | 2D spatial grid cell timeseries for 400 grid cells over 48 hours. |
| spatial_emission_inventory.csv | Stage 5 | Grid-allocated anthropogenic emissions (PM2.5, PM10, NOx, CO) in kg/s. |
| biomass_fire_emissions.csv | Stage 6 | Satellite active fire detections, FRP, biomass burned, and pollutant emissions. |
| chemistry_summary.csv | Stage 8 | Summary min/max/mean of gas-phase species (O3, NO, NO2, CO, SO2, PM2.5). |
| aerosol_summary.csv | Stage 8 | Summary of aerosol mass (PM2_5_DRY) and aerosol optical depth (AOD550). |
| feedback_summary.csv | Stage 9 | Quantified feedback response table comparing CONTROL (Feedback OFF) vs FEEDBACK (Feedback ON). |
| delta_pm25.png, delta_pblh.png, delta_t2.png, delta_swdown.png | Stage 9 | Spatial 2D difference maps illustrating aerosol radiative forcing and PBL trapping. |
| forecast_fire_emissions.csv | Stage 10 | 72-hour projected fire emissions incorporating half-life persistence decay. |
| regional_forecast.csv | Stage 10 | 72-hour regional pollutant concentration forecast timeseries. |
| forecast.db | Stage 10 & 11 | SQLite database storing forecast runs, 72h predictions, observations, and alerts. |
| validation_24h.csv, validation_48h.csv, validation_72h.csv | Stage 10 | Lead-time horizon validation skill metrics (PM2.5 24h MAE = 20.29, 48h MAE = 29.74, 72h MAE = 34.04). |
| forecast_pm25.png ... forecast_co.png | Stage 10 | 72-hour forecast trend graphs for all primary pollutants. |